## RCA Categories

Construction professionals investigate a relatively consistent set of RCA categories. For insurance claims, the highest-priority causes are usually:

- Defective products, materials, or components

- Storm and natural-catastrophe damage

- Faulty workmanship and maintenance

- Water damage and leakage

- Fire and explosion

- Design errors and engineering omissions

- Equipment or machinery failure

- Theft, vandalism, and site security failures

- Construction accidents and unsafe work practices

- Schedule, coordination, and change-management failures

The ranking changes depending on whether “importance” means number of claims, total dollar loss, fatality risk, legal exposure, or project-delay impact.

The best quantified benchmark I found is Allianz’s analysis of 13,599 engineering and construction insurance claims from July 2013 through July 2018, valued at approximately €7.9 billion. It is not a universal construction-industry dataset, but it is useful for prioritization.1

Ranking by insurance-claim frequency and severity

| Combined priority | RCA cause category | Share by claim count | Share by claim value | Why professionals investigate it |
| --- | --- | --- | --- | --- |
| 1 | Defective products and materials | 27% | 17% | High frequency, major rework, latent defects, failed components, and supplier/subcontractor responsibility |
| 2 | Storm and natural-catastrophe damage | 10% | 10% | Weather can damage large exposed areas and may trigger major delay, mitigation, and coverage questions |
| 3 | Faulty workmanship and maintenance | 9% | 15% | Often causes recurring defects, water intrusion, structural issues, and disputes over responsibility |
| 4 | Water damage and leakage | 8% | 4% | Very common, can spread through concealed areas, and requires difficult source-and-timing analysis |
| 5 | Fire and explosion | 7% | 27% | Less frequent than defects or storms, but the most severe category by total claim value |
| 6 | Design and engineering errors | Included within broader defect categories | Included within broader defect categories | Can cause rework, delay, structural problems, code issues, and professional-liability claims |
| 7 | Equipment and machinery failures | Not separately quantified in the cited Allianz table | Not separately quantified | Can cause property damage, injury, interruption, and questions about maintenance or operator error |
| 8 | Theft, vandalism, and site-security failures | Not separately quantified | Not separately quantified | Common during partially secured construction and may involve inventory, access, and security-control failures |
| 9 | Construction accidents and unsafe conditions | Often handled under workers’ compensation or general liability rather than engineering property claims | Not directly comparable | High injury, fatality, liability, and regulatory exposure |
| 10 | Schedule, coordination, and change-management failures | Usually embedded in delay, professional liability, or builder’s-risk claims | Often difficult to isolate | Can create delay, acceleration cost, concurrent-delay disputes, and business-interruption losses |

Allianz’s figures show an important distinction: defective products are the most frequent engineering-claim category at 27%, while fire and explosion produce the largest share of loss value at 27%. Allianz reports that faulty workmanship and maintenance account for 9% of claim count and 15% of value, water damage for 8% and 4%, and storm damage for 10% and 10%.1

Together, defective products and faulty workmanship account for 36% of the claim count and 32% of claim value in that Allianz analysis. The two percentages are calculated from the published categories and are useful as directional indicators, not universal market constants.

The most important RCA categories in practice

1. Defective materials, products, and components

This is the highest-ranked category by claim frequency in the Allianz data. Investigators typically ask:

- Was the material approved?

- Was the correct product delivered?

- Was it stored correctly?

- Was it damaged before installation?

- Did it meet specifications?

- Was there a manufacturing defect?

- Were substitutions approved?

- Was installation compatible with the product?

- Were testing and commissioning completed?

- Did the same product fail elsewhere?

Typical evidence includes submittals, procurement records, delivery tickets, certificates, laboratory tests, photographs, manufacturer instructions, and samples from the failed installation.

2. Storm and natural-catastrophe damage

Storms are a leading claim category by frequency and value. RCA usually examines:

- Wind speed and direction.

- Rainfall intensity.

- Hail, lightning, flood, freeze, or wildfire exposure.

- Site drainage and temporary protection.

- Roof and façade completion status.

- Openings, penetrations, and temporary enclosures.

- Weather warnings and site shutdown decisions.

- Whether the damage resulted from the storm itself or inadequate protection against a foreseeable event.

This category is especially relevant to visual documentation because a time series can show whether roofs, windows, barriers, drainage, and temporary coverings were installed before the event.

3. Faulty workmanship and maintenance

This category is more than “someone installed it incorrectly.” Investigators usually distinguish:

- Incorrect installation.

- Incomplete installation.

- Work outside the approved method.

- Poor sequencing.

- Failure to inspect.

- Failure to protect completed work.

- Deficient maintenance.

- Failure to correct a previously reported issue.

- Inadequate supervision or competency.

The key question is often whether the damage was caused by one defective act or by a broader management-control failure.

4. Water damage and leakage

Water damage ranks high by frequency and is one of the most difficult categories to investigate because the visible damage often occurs away from the source. Common competing causes include:

- Roof or membrane failure.

- Temporary roof-protection failure.

- Plumbing or sprinkler leak.

- Failed pressure test.

- Unsealed window or façade opening.

- Flooding or groundwater.

- Blocked or undersized drainage.

- Condensation.

- Concrete curing water.

- Improper sequencing.

- Water introduced during cleaning or remediation.

- Pre-existing moisture.

Chubb specifically emphasizes that builders-risk coverage may distinguish between water damage, flood, storm surge, and other sources, and that even a slow leak can produce a large claim.2 For RCA, the crucial evidence is usually temporal and spatial: the last known dry condition, first known wet condition, source location, moisture path, weather, testing records, and repair history.

5. Fire and explosion

Fire is the most important category by claim value in Allianz’s engineering-claim analysis. Construction fire RCA commonly examines:

- Hot work.

- Welding and cutting.

- Temporary electrical systems.

- Temporary heating.

- Fuel and flammable-liquid storage.

- Fire watch and impairment procedures.

- Combustible materials.

- Fire protection availability.

- Smoking controls.

- Battery charging.

- Arson or unauthorized access.

- Emergency response time.

Because fire can destroy evidence, pre-loss photographs, inspection logs, hot-work permits, security records, and time-stamped site imagery can be particularly important.

6. Design errors and engineering omissions

Design problems may appear as defects, but professionals often investigate them separately because the responsible party, insurance coverage, and corrective action can differ. Common examples include:

- Incorrect structural design.

- Inadequate drainage.

- Missing waterproofing details.

- Incompatible mechanical and architectural systems.

- Insufficient clearances.

- Incomplete drawings.

- Conflicting specifications.

- Uncoordinated MEP design.

- Unclear tolerances.

- Failure to account for construction sequencing.

- Design changes issued after work began.

The key evidence usually includes drawings, revisions, RFIs, submittals, design-review comments, calculations, meeting minutes, and instructions from the architect or engineer.

7. Equipment and machinery failure

This category includes both equipment-caused property losses and worker injuries. Investigators examine:

- Maintenance and inspection records.

- Operator qualifications.

- Manufacturer instructions.

- Load charts and lift plans.

- Telematics.

- Equipment position.

- Weather and ground conditions.

- Component failure.

- Guarding and safety devices.

- Whether a known defect was left uncorrected.

- Whether the equipment was used outside its intended purpose.

For claims involving cranes, lifts, vehicles, and heavy equipment, the causal chain often includes equipment condition, operator action, planning, supervision, and site layout.

8. Theft, vandalism, and site-security failures

These claims usually require investigation of:

- Fencing and access points.

- Lighting.

- Cameras and alarms.

- Guard services.

- Badge or key records.

- Inventory and delivery records.

- Last-known visual condition.

- Temporary storage.

- Whether materials were attractive targets.

- Whether security controls were operational.

The relevant RCA question is often not only “who took the material?” but also “which security-control failure allowed the loss?”

9. Worker injury and unsafe conditions

For workers’ compensation and general-liability claims, the most serious construction hazards remain the OSHA “Focus Four”:

- Falls.

- Struck-by incidents.

- Electrocutions.

- Caught-in or caught-between incidents.

For 2021, the Texas Department of Insurance reports falls at 37% of construction deaths, electrocution at approximately 8%, struck-by incidents at approximately 8%, and caught-in/between incidents at approximately 5%.3 These percentages are fatality statistics, not insurance-claim frequencies.

The RCA investigation normally covers:

- Task and work activity.

- Site configuration.

- PPE and fall protection.

- Training and competency.

- Supervision.

- Job hazard analysis.

- Permit records.

- Equipment condition.

- Weather and visibility.

- Prior observations or near misses.

- Whether a known hazard was reported and left unresolved.

10. Schedule, coordination, and change-management failures

These may not appear as a single insurance “cause of loss,” but they are central to delay and construction-dispute RCA. Investigators examine:

- Late design information.

- RFI response time.

- Submittal delays.

- Material procurement.

- Labor availability.

- Access constraints.

- Trade interference.

- Rework.

- Change orders.

- Owner-directed changes.

- Inspection or approval delays.

- Weather.

- Incomplete predecessor work.

- Concurrent delays.

- Poor schedule logic.

- Failure to mitigate or accelerate.

The key distinction is between:

text

event that made work harder

and:

text

event that actually delayed the critical path and moved project completion

Different rankings for different insurance lines

Builders-risk and construction-property claims

Prioritize:

- Fire and explosion.

- Storm and natural catastrophe.

- Water damage.

- Defective products and materials.

- Faulty workmanship.

- Theft and vandalism.

- Equipment impact or failure.

- Design-related physical damage.

- Collapse or structural failure.

- Delay and soft-cost impact.

General liability and construction-defect claims

Prioritize:

- Faulty workmanship.

- Defective materials.

- Design and engineering errors.

- Water intrusion and envelope failure.

- Structural or foundation defects.

- MEP coordination failures.

- Code or specification noncompliance.

- Site conditions and adjacent-property damage.

- Contractor or subcontractor negligence.

- Failure to maintain or correct known conditions.

Workers’ compensation and employer-liability claims

Prioritize:

- Falls.

- Struck-by incidents.

- Caught-in/between incidents.

- Electrocution.

- Equipment and vehicle incidents.

- Scaffolding and ladder failures.

- Excavation and trench collapse.

- Manual handling and ergonomic injuries.

- Exposure to silica, chemicals, noise, or heat.

- Inadequate training, supervision, or hazard communication.

Delay, builder’s-risk time-element, and professional-liability claims

Prioritize:

- Design changes and late design information.

- Defective work and rework.

- Water, fire, or storm damage.

- Material and equipment procurement.

- Access, sequencing, and trade coordination.

- Inspection and approval delays.

- Labor and subcontractor performance.

- Weather and site conditions.

- Change-order administration.

- Concurrent delay and mitigation disputes.

Recommended ranking for an RCA model

For the multimodal RCA system you described, I would prioritize the training and evaluation categories in this order:

| Rank | Model category | Why it should be prioritized |
| --- | --- | --- |
| 1 | Water intrusion and leakage | High practical frequency, strong visual component, many competing causes, and direct insurance relevance |
| 2 | Faulty workmanship and installation | Common across defects, quality, water, and delay claims |
| 3 | Defective materials or products | Highest frequency in Allianz’s engineering-claim analysis |
| 4 | Fire and hot-work events | Lower frequency but very high severity and strong evidence requirements |
| 5 | Storm and temporary-protection failures | Good fit for time-series visual evidence and weather integration |
| 6 | Design and coordination errors | Requires multimodal document reasoning across drawings, RFIs, and site conditions |
| 7 | Equipment and machinery incidents | Combines visual, maintenance, operator, and telemetry evidence |
| 8 | Safety incidents | Strong construction relevance and established hazard taxonomies |
| 9 | Theft and security failures | Strong temporal and access-control evidence use case |
| 10 | Schedule and change-management causes | Highest value for delay RCA, but requires integrating P6 and project records rather than photos alone |

Important qualification

There is no single universal ranking of “claims filed” across all construction insurance. The categories differ by:

- Builders-risk versus workers’ compensation.

- General liability versus professional liability.

- Property damage versus bodily injury.

- Claim count versus claim value.

- Country and regulatory system.

- Project type and construction phase.

- Whether the dataset includes disputed or only paid claims.

Therefore, the most defensible summary is:

Defective products and workmanship are frequent; storm and water losses are common and highly investigable; fire is less frequent but disproportionately severe; falls and the other Focus Four dominate worker-fatality RCA; design, coordination, and schedule failures dominate complex delay and defect disputes.

Footnotes

- Allianz Commercial, “Engineering & Construction Claims and Insurance Trends” ↩ ↩2

- Chubb, “Builders Risk Insurance: Water Damage or Flood?” ↩

- Texas Department of Insurance, “OSHA’s Fatal Four” ↩

## Building sensors

Yes. Building sensors can significantly strengthen water-damage evidence, especially when combined with time-stamped 360° site imagery, schedules, weather, inspection records, and repair documentation.

Photos show what the site looked like. Sensors can show when conditions changed, how fast they changed, and whether water was actively flowing.

What sensors can contribute

1. Point-of-leak sensors

These detect direct contact with water at specific locations, such as:

- Mechanical rooms.

- Elevator pits and shafts.

- Plumbing risers.

- Under temporary plumbing connections.

- Near sprinkler systems.

- Below roof penetrations.

- Around high-value electrical equipment.

- At basement or below-grade locations.

They can establish:

text

sensor_id

location

first wet timestamp

last dry timestamp

duration wet

alert timestamp

acknowledgment timestamp

This is valuable for bounding the onset of an event. It still does not necessarily identify the upstream cause.

2. Flow meters

Flow sensors monitor water volume through a main line, riser, zone, or temporary construction supply.

They can identify:

- Sudden pipe rupture.

- Continuous overnight flow.

- Abnormally high use.

- Flow when the site should be unoccupied.

- A low-flow leak that continues for hours.

- Whether water use stopped after a valve closed.

Nationwide’s construction guidance describes both point sensors and flow-based detection. Point sensors detect contact with water, while flow systems identify abnormal water-use patterns.1

Flow data are particularly useful for distinguishing:

text

interior pipe release → abnormal flow on a plumbing zone

weather intrusion → rainfall without corresponding interior flow anomaly

That distinction is not conclusive, but it helps prioritize investigation.

3. Temperature sensors

Temperature sensors can help identify:

- Freeze risk.

- Conditions preceding a burst pipe.

- Cold zones around temporary enclosures.

- Failure of temporary heating.

- Temperature recovery after a heating-system repair.

A temperature trend can support a hypothesis such as:

text

temperature fell below threshold

→ pipe froze

→ pipe burst

→ water sensor activated

It does not prove that freezing caused the failure unless combined with physical inspection.

4. Relative-humidity sensors

Humidity sensors help detect:

- Persistent moisture.

- Conditions favorable to mold growth.

- Wetting before visible damage appears.

- Inadequate drying after a loss.

- Condensation risk.

- Whether an area returned to normal conditions after remediation.

Humidity is usually indirect evidence. High humidity can result from water intrusion, poor ventilation, concrete curing, weather, or drying operations. It should not be treated as proof of a leak.

5. Moisture-content and in-wall sensors

Moisture meters and embedded sensors can help monitor:

- Drywall.

- Wood framing.

- Insulation.

- Concrete slabs.

- Roof assemblies.

- Envelope components.

- Areas behind finishes or above ceilings.

These are useful for measuring the extent and persistence of damage. They can also show whether moisture existed before a reported event, although this requires careful calibration and historical data.

6. Automatic shutoff systems

Automatic shutoff valves do not merely collect evidence. They also mitigate loss by stopping flow.

They can provide event records such as:

text

valve_open

abnormal flow detected

valve closed automatically

valve manually overridden

flow resumed

Zurich describes construction-site systems that combine point sensors, flow sensing, temperature monitoring, automatic shutoff, backup power, and building-management-system integration.2

Construction water events should be classified into three categories

A useful RCA system should distinguish:

| Category | Example | Best sensor evidence |
| --- | --- | --- |
| Intrusion | Rain through roof, façade, window, or temporary opening | Moisture/envelope sensors, humidity, rainfall, visual imagery |
| Release | Burst pipe, failed fitting, sprinkler discharge, plumbing leak | Flow meters, point sensors, pressure data, shutoff logs |
| Inundation | Groundwater, floodwater, or water entering through below-grade paths | Sump-level sensors, basement water sensors, rainfall, groundwater and drainage data |

This distinction is important because each category requires different sensor placement and different causal tests. Wint’s construction guidance uses the same three-way distinction and notes that below-grade inundation is often overlooked despite the value of basement electrical and mechanical equipment.3

How sensors and visual data work together

The strongest evidence package joins every sensor event to the visual site record:

text

sensor event

→ physical location

→ nearest 360° capture

→ floor-plan/BIM element

→ active P6 activity

→ weather and site conditions

→ inspection or repair record

Example:

text

April 14, 02:13

Flow sensor: Level 4 plumbing zone shows continuous flow

April 14, 02:18

Point sensor: Mechanical room sensor becomes wet

April 14, 09:00

360° capture: water visible below a pipe riser;

roof area appears dry

April 14, 10:30

Pressure test: failed connection identified

Conclusion:

Probable interior plumbing release, not roof intrusion

Another example:

text

April 7

360° capture: roof penetration temporarily protected

April 11

Weather station: heavy rainfall

April 12, 03:40

Humidity rises in Level 4 east wing;

no abnormal interior water flow

April 14

360° capture: wet insulation and ceiling staining below penetration

Conclusion:

Probable exterior water intrusion;

exact entry point requires roof inspection

The sensors do not replace the photos. They provide time-series and physical-state evidence that makes the visual evidence more interpretable.

Recommended data structure

For every sensor event, store:

JSON

{

"event_id": "WATER-0414-001",

"sensor_id": "FLOW-L4-EAST",

"sensor_type": "flow_meter",

"location": {

"building": "Main",

"level": "04",

"zone": "East",

"bim_element": "RISER-04-12"

},

"timestamp_start": "2026-04-14T02:13:00Z",

"timestamp_end": "2026-04-14T02:46:00Z",

"baseline_value": 0,

"observed_value": 18.2,

"unit": "liters_per_minute",

"threshold": 5.0,

"alert_sent_at": "2026-04-14T02:14:00Z",

"valve_closed_at": "2026-04-14T02:18:00Z",

"nearest_visual_captures": [

"CAP-0413-LEVEL4",

"CAP-0414-LEVEL4"

],

"candidate_activities": [

"A-7310",

"A-7420"

]

}

Important metadata include:

- Calibration date.

- Sensor accuracy and detection threshold.

- Battery and connectivity status.

- Time synchronization source.

- Installation date.

- Whether the sensor was relocated.

- Whether it was functioning during the event.

- Alert recipients.

- Response and acknowledgment timestamps.

- Original data versus corrected or interpolated data.

Sensor placement should be risk-based

Do not distribute sensors uniformly across the site. Prioritize:

- Main water entries and risers.

- Temporary water lines.

- Mechanical rooms.

- Elevator pits.

- Below-grade electrical rooms.

- Areas with moisture-sensitive materials.

- Roof penetrations and incomplete envelope areas.

- Plumbing-intensive zones.

- Areas with high-value installed equipment.

- Locations where a leak could remain hidden.

Flow systems should be divided into meaningful zones. A building-wide meter can miss a small leak because the normal project flow is too high. Tighter zoning improves sensitivity to low-flow events.3

Sensor evidence is not automatically causal evidence

A sensor can show:

text

water was present at Location A at Time T

It usually cannot independently show:

text

the roof membrane failed and caused the water

For causal RCA, sensor evidence should be combined with:

- Visual evidence.

- Rainfall and weather.

- Pressure or leak testing.

- Plumbing and sprinkler records.

- Roof and envelope inspection.

- Work permits.

- Daily reports.

- Contractor statements.

- Prior sensor history.

- Moisture-path analysis.

- Repair and recurrence data.

A flow spike in a plumbing zone supports an interior-release hypothesis. It does not establish whether the cause was a defective fitting, accidental damage by a trade, improper installation, failed pressure test, or a valve left open.

Sensors improve model training

For a multimodal RCA model, sensor data provide several benefits.

Better temporal labels

Instead of only:

text

dry on April 7, wet on April 14

you may have:

text

humidity began rising: April 11, 22:04

flow anomaly began: April 14, 02:13

point sensor activated: April 14, 02:18

visual damage first documented: April 14, 09:00

This helps train onset detection and event-sequence reasoning.

Better competing-cause discrimination

The model can compare:

| Hypothesis | Supporting pattern |
| --- | --- |
| Plumbing release | Flow anomaly, point sensor activation, no rainfall required |
| Roof intrusion | Rainfall, humidity rise, no interior flow anomaly, spatial alignment below roof |
| Freeze burst | Temperature drop followed by flow spike and wet sensor |
| Condensation | High humidity, temperature differential, slow moisture increase, no discrete flow event |
| Groundwater inundation | Basement-level rise, rainfall or drainage data, no plumbing-zone anomaly |

Better “unknown” cases

If sensors show moisture but there is no source evidence, the correct label may be:

text

water damage confirmed; cause unresolved

These cases are extremely important for preventing overconfident model behavior.

Key limitations

Sensors introduce their own evidence risks:

- Sensor failure or dead batteries.

- Poor placement.

- Connectivity loss.

- Clock drift.

- False positives from cleaning or testing.

- False negatives from low-flow leaks.

- Sensors moved during construction.

- Flow thresholds set too high.

- Humidity affected by normal curing.

- Data gaps during power outages.

- Incomplete coverage of concealed spaces.

- Unclear calibration history.

- Alerts received but not acted upon.

A missing sensor alert does not prove that no water was present.

Human validation remains important

Sensors reduce uncertainty, but they do not eliminate the need for expert review in disputed or high-value cases. A human should validate:

- Whether the sensor was operating correctly.

- Whether the sensor location matches the claimed loss area.

- Whether the readings are physically plausible.

- Whether the signal could have another explanation.

- Whether the sensor event corresponds to the reported damage.

- Whether the event caused a schedule impact.

- Whether competing sources were adequately excluded.

For automated workflows, the model can classify cases into:

- High-confidence interior release

- High-confidence external intrusion

- Possible freeze event

- Possible condensation

- Sensor evidence inconsistent with reported cause

- Insufficient evidence

- Requires physical inspection

Best architecture for your use case

text

360° imagery and photos

+

water-flow sensors

+

point leak sensors

+

temperature and humidity

+

moisture-content sensors

+

weather data

+

P6 schedule

+

daily reports, tests, RFIs, and repairs

↓

time-synchronized evidence graph

↓

visual and sensor event detection

↓

candidate source classification

↓

competing-cause ranking

↓

affected activity and delay mapping

↓

human validation for consequential findings

Bottom line

Yes—building sensors can be one of the most valuable additions to a visual construction evidence platform. They provide continuous, objective, time-stamped information that photos typically lack. The best combination is:

- 360° imagery for spatial and visual context.

- Flow sensors for internal water-release events.

- Point sensors for localized detection.

- Temperature sensors for freeze risk.

- Humidity and moisture sensors for persistence and hidden damage.

- Weather and schedule data for causal and delay analysis.

Sensors should be treated as causal evidence inputs, not as automatic proof of cause. Their greatest value is narrowing the timeline and separating competing mechanisms so that a model—and ultimately a human investigator—can reach a more defensible conclusion.

Footnotes

- IRMI/Nationwide, “Sensors and Technology Can Protect Your Construction Project” ↩

- Zurich Resilience Solutions, “Water Leak Detection Sensors” ↩

- Wint, “How to Set Up Water Monitoring Technology on a Construction Site” ↩ ↩2

## References

1. Root-cause analysis methods and commercial tools

- ECI Solutions — Manufacturing Root Cause Analysis Tools

- TapRooT — Root Cause Analysis Software

- TapRooT — Construction Solutions

- EcoOnline — Root Cause Analysis Tools

- EHS Insight — Root Cause Analysis Software

- EHS Insight — Construction Safety Management Software

- Procore — Configure Incidents Root Cause Analysis Form

- Procore — Quality and Safety

- Autodesk Construction Cloud — Construction Site Safety

2. Open datasets and investigation corpora

- ConstructCIE GitHub Repository

- OSHA Construction Engineering Reports

- NIST Disaster and Failure Studies

- CPWR Construction FACE Database

- Construction Safety Dataset

- Mendeley — BIM-Context-Aware Evidence-Traceable Safety Review Dataset

- MSHA Fatality Reports Search

- NTSB Accident Docket Search

- World Bank Projects and Operations Dataset

- Asian Development Bank Project Completion Report Example

- Mendeley — Dataset of Bridge Collapses in Italy

- Work Zone Data Exchange Feed Registry

- NYC Change Orders: Cycle Time and Volume

- Mendeley — Dataset of Causes of Delays in Construction Projects

3. Primavera P6 and schedule integration

- Oracle P6 EPPM REST API — Read Activities

- Oracle P6 EPPM REST API — Activity Endpoints

- Oracle P6 EPPM REST API — All REST Endpoints

- Oracle P6 EPPM REST API — Relationship Endpoints

- Oracle P6 — Relationships Tab

- Oracle Primavera Cloud REST API

4. Construction visual documentation and reality capture

- OnSiteIQ — Why 360° Construction Photo Documentation Is Critical

- OnSiteIQ

- OnSiteIQ — Construction Documentation Platform

- Insta360 — Construction 360 Jobsite Capture

- DroneDeploy — 360 Site Capture and Construction Documentation

- OpenSpace — Reality Capture 101

- OpenSpace — 360° Construction Documentation

- RECONSTRUCT — 360-Degree Construction Reality Mapping

- RECONSTRUCT — Reality Capture and Mapping for Construction

- Matterport — Reality Capture for Construction

- Track3D — Reality Capture Best Practices

5. Construction insurance, underwriting, and claims evidence

- AXA XL — Building a Great Builder’s Risk Submission

- J.S. Held — Builder’s Risk Project Documentation

- J.S. Held — Strategic Management of Builder’s Risk Claims

- Crum & Forster — Risk Engineering for Contractors

- Texas Department of Insurance — Construction Safety Inspection Checklist PDF

- Allianz Commercial — Engineering and Construction Claims Trends

- Allianz — Engineering and Construction Risk Trends PDF

- Chubb — Builder’s Risk Insurance: Water Damage or Flood?

- US Assure — Water Intrusion and Flood Coverage

- Independent Agent Magazine — Three Real-Life Builder’s Risk Claims

- Envista Forensics — Builder’s Risk Losses

- HKA — Forensic Engineering for Construction Claims

- Salas O’Brien — Forensic Analysis After a Disaster

- CLM Magazine — Forensic Investigation in Construction Defect Claims

- Rimkus — Forensic Investigation in Defective Work Claims

- HPM Consultants — Case Studies

- Hendershot Cowart — Construction Delay Dispute Case Study

- IAARC — Construction Delay Impact Analysis PDF

6. Construction accident and safety RCA

- OSHA — Construction Focus Four

- OSHA — Construction Fall Protection

- OSHA — Electrical Safety in Construction

- Texas Department of Insurance — OSHA Fatal Four

- Rimkus — Top 10 Causes of Construction Accidents

- OSHA — Electrocution Hazards PDF

- OSHA — Caught-In or Caught-Between Hazards PDF

- OSHA — Excavation and Trenching

- OSHA — Top 10 Most Frequently Cited Standards

- BLS — Fatal Work Injuries in 2024

- BLS — Fatal Occupational Injuries Table A-1

7. Water damage, sensors, and monitoring

- IRMI/Nationwide — Sensors and Technology for Construction Projects

- Zurich Resilience Solutions — Water Leak Detection Sensors

- Wint — Water Monitoring Technology on Construction Sites

- AIG — Water Damage Prevention Program for Construction PDF

- Nationwide — Water Damage Response Plan PDF

- CIREG / AquaLeak — Construction Water Leak Detection

- Alert Labs — Construction Water Management

- EYrus — Construction Humidity Monitoring

- FM — Water Damage and Escaped Liquids

- Zurich — Water Damage Mitigation Brochure PDF

- Zurich — Water Damage Mitigation Fact Sheet PDF

- Chubb — Preventing Water Damage

- Chubb — Smart Water-Damage Prevention

- Nationwide — Construction Water Mitigation Program

8. Construction-defect and claims references

- J.S. Held — Construction Defect Insurance Claims

- The Advocate Magazine — Construction-Defect Coverage Issues

- Scarinci Hollenbeck — Causes of Construction Defect Litigation

- HD Law — Construction Defect Claims

- Hollington Law — Types of Construction Defect Claims

9. Other links that appeared during research

- Insurance Business — Key Causes of Engineering and Construction Insurance Claims

- Crum & Forster — Contractor Risk Engineering

- The Hartford — Loss Control Services

- OnArchipelago — Risk Engineering for Insurance Brokers

- Aon — Insurance Claims Trends

- Rimkus — Vehicle Accident Reconstruction

- IA Magazine — Independent Insurance Agents

## Datasets

https://medium.com/ai4sm/exploratory-spatial-data-analysis-for-smart-mobility-root-cause-analysis-717e76bb6c19

https://www.irmi.com/articles/expert-commentary/sensors-and-technology-can-protect-your-construction-project

ConstructionCIE

https://arxiv.org/html/2608.06495v4

https://huggingface.co/datasets/lab-flair/constructcie